# Lab Report 11

## Assignment 1

Code:

.eqv IN_ADDRESS_HEXA_KEYBOARD       0xFFFF0012

# receive row and column of the key pressed, 0 if not key pressed

# Eg. equal 0x11, means that key button 0 pressed.

# Eg. equal 0x28, means that key button D pressed.

.eqv OUT_ADDRESS_HEXA_KEYBOARD      0xFFFF0014

.text

main:

li      $t1,        IN_ADDRESS_HEXA_KEYBOARD

li      $t2,        OUT_ADDRESS_HEXA_KEYBOARD

polling:

check01:

li      $t3,        0x01                                # check 0, 1, 2, 3

sb      $t3,        0($t1)                              # must reassign expected row

lbu     $a0,        0($t2)                              # read scan code of key button

bne     $a0,        0,                          print

check02:

li      $t3,        0x02                                # check 4, 5, 6, 7

sb      $t3,        0($t1)                              # must reassign expected row

lbu     $a0,        0($t2)                              # read scan code of key button

bne     $a0,        0,                          print

check03:

li      $t3,        0x04                                # check 8, 9, a, b

sb      $t3,        0($t1)                              # must reassign expected row

lbu     $a0,        0($t2)                              # read scan code of key button

bne     $a0,        0,                          print

check04:

li      $t3,        0x08                                # check c, d, e, f

sb      $t3,        0($t1)                              # must reassign expected row

lbu     $a0,        0($t2)                              # read scan code of key button

print:

li      $v0,        34                                  # print integer (hexa)

syscall

sleep:

li      $a0,        1000                                # sleep 1000ms

li      $v0,        32

syscall

back_to_polling:

j       polling                                         # continue polling

Results:

Comments:

- Because of the polling technique, there are no activities recorded in Coproc 0.

## Assignment 2

Comments:

- Set a breakpoint for the interruption.

- After enabling interruption, the code will keep sleeping until an interruption happens.

- If an interruption happens, the execution will pause at the specified breakpoint.

- Here, $12 = 0x0000ff13, $13 = 0x00000800, $14 = 0x00400024.

- After this, the program prints the message and updates $14 to 0x00400028 and jump to code at the address 0x00400028.

## Assignment 3

Code:

.eqv IN_ADDRESS_HEXA_KEYBOARD 0xFFFF0012

.eqv OUT_ADDRESS_HEXA_KEYBOARD 0xFFFF0014

.data

Message: .asciiz "Key scan code "

# ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# MAIN Procedure

# ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.text

main:

# ---------------------------------------------------------

# Enable interrupts you expect

# ---------------------------------------------------------

# Enable the interrupt of Keyboard matrix 4x4 of Digital Lab Sim

li      $t1,    IN_ADDRESS_HEXA_KEYBOARD

li      $t3,    0x80                                    # bit 7 = 1 to enable

sb      $t3,    0($t1)

# ---------------------------------------------------------

# Loop an print sequence numbers

# ---------------------------------------------------------

xor     $s0,    $s0,                        $s0         # count = $s0 = 0

Loop:

addi    $s0,    $s0,                        1           # count = count + 1

prn_seq:

addi    $v0,    $zero,                      1

add     $a0,    $s0,                        $zero       # print auto sequence number

syscall

prn_eol:

addi    $v0,    $zero,                      11

li      $a0,    '\n'                                    # print endofline

syscall

sleep:

addi    $v0,    $zero,                      32

li      $a0,    300                                     # sleep 300 ms

syscall

nop                                                     # WARNING: nop is mandatory here.

b       Loop                                            # Loop

end_main:

# ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

# GENERAL INTERRUPT SERVED ROUTINE for all interrupts

# ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

.ktext 0x80000180

# -------------------------------------------------------

# SAVE the current REG FILE to stack

# -------------------------------------------------------

IntSR:

addi    $sp,    $sp,                        4           # Save $at because we may change it later

sw      $at,    0($sp)

addi    $sp,    $sp,                        4           # Save $sp because we may change it later

sw      $v0,    0($sp)

addi    $sp,    $sp,                        4           # Save $a0 because we may change it later

sw      $a0,    0($sp)

addi    $sp,    $sp,                        4           # Save $t1 because we may change it later

sw      $t1,    0($sp)

addi    $sp,    $sp,                        4           # Save $t3 because we may change it later

sw      $t3,    0($sp)

# --------------------------------------------------------

# Processing

# --------------------------------------------------------

prn_msg:

addi    $v0,    $zero,                      4

la      $a0,    Message

syscall

check01:

li      $t1,    IN_ADDRESS_HEXA_KEYBOARD

li      $t3,    0x81                                    # check row 1 and re-enable bit 7

sb      $t3,    0($t1)                                  # must reassign expected row

li      $t1,    OUT_ADDRESS_HEXA_KEYBOARD

lbu     $a0,    0($t1)

bne     $a0,    0,                          prn_cod

check02:

li      $t1,    IN_ADDRESS_HEXA_KEYBOARD

li      $t3,    0x82

sb      $t3,    0($t1)                                  # must reassign expected row

li      $t1,    OUT_ADDRESS_HEXA_KEYBOARD

lbu     $a0,    0($t1)

bne     $a0,    0,                          prn_cod

check03:

li      $t1,    IN_ADDRESS_HEXA_KEYBOARD

li      $t3,    0x84

sb      $t3,    0($t1)                                  # must reassign expected row

li      $t1,    OUT_ADDRESS_HEXA_KEYBOARD

lbu     $a0,    0($t1)

bne     $a0,    0,                          prn_cod

check04:

li      $t1,    IN_ADDRESS_HEXA_KEYBOARD

li      $t3,    0x88

sb      $t3,    0($t1)                                  # must reassign expected row

li      $t1,    OUT_ADDRESS_HEXA_KEYBOARD

lbu     $a0,    0($t1)

prn_cod:

li      $v0,    34

syscall

li      $v0,    11

li      $a0,    '\n'                                    # print end of line

syscall

# --------------------------------------------------------

# Evaluate the return address of main routine

# epc <= epc + 4

# --------------------------------------------------------

next_pc:

mfc0    $at,    $14                                     # $at <= Coproc0.$14 = Coproc0.epc

addi    $at,    $at,                        4           # $at = $at + 4 (next instruction)

mtc0    $at,    $14                                     # Coproc0.$14 = Coproc0.epc <= $at

# --------------------------------------------------------

# RESTORE the REG FILE from STACK

# --------------------------------------------------------

restore:

lw      $t3,    0($sp)                                  # Restore the registers from stack

addi    $sp,    $sp,                        -4

lw      $t1,    0($sp)                                  # Restore the registers from stack

addi    $sp,    $sp,                        -4

lw      $a0,    0($sp)                                  # Restore the registers from stack

addi    $sp,    $sp,                        -4

lw      $v0,    0($sp)                                  # Restore the registers from stack

addi    $sp,    $sp,                        -4

lw      $at,    0($sp)                                  # Restore the registers from stack

addi    $sp,    $sp,                        -4

return:

eret                                                    # Return from exception

Results:

Comments:

- Clicking a button will interrupt the program and print the button code to the console.

- Set a breakpoint to see changes in Coproc 0.

- Here, clicking 5 will make the program output “Key scan code 0x00000022”.

- In Coproc 0, $12 = 0x0000ff13, $13 = 0x00000800, $14 = 0040003c.

## Assignment 4

Comments:

- Set a breakpoint for each interruption.

- If time counter interruption happens, $13 = 0x00000400 and the program outputs “Time interval!”.

- If key matrix interruption happens, $13 = 0x00000800 and the program outputs “Someone has pressed a key!”.

## Assignment 5

Results:

Comments:

- Set breakpoint for the interruption.

- If the interruption happens, $13 = 0x00000034 and the program outputs the encoded character.